SOLIGOR REVERSER RING            For Miranda Camera

It is an optical fact that normal lenses when used for close-up photography perform best if they are reversed.    The Soligor Reverser Ring permits you to reverse your normal lens, and in this way obtain the sharpest image,

INSTRUCTIONS

It can be used three ways:

1     The first method requires only a Miranda camera body, 50mm lens, and adapter.    The closest focusing distance of the Miranda without any accessories is 17 inches.    With the Reverser Adapter, you can get as close as 4. 66 inches (118. 5mm) when lens is set at 17 inches.    Magnification of 0. 75 X is achieved with a field of view of 40mm x 58mm.    To use the adapter, remove the lens, screw in the adapter with the legend "Reverser Ring for Miranda" facing the camera body, and thread the lens on to the adapter. Because the lens is now further away from the film plane than in the normal position, you must calculate a 3.17X exposure factor.    You can obtain the correct exposure by either dividing the exposure factor into the film speed or opening the lens a fraction more than one and a half (1. 6) stops as indicated by your meter reading.

- The Adapter can also be used on the Miranda Extension Tubes.   This set of extension tubes permits you to focus even closer.    Follow the same instructions as above, but instead screw the adapter into the extension tube instead of the camera body, and then the tube into the camera body.    Since different lengths of the tubes can be used, different exposure factors are needed.    Consult tube instructions, adding a 3.17X exposure factor to the tube factor.

- The third way to use the Soligor Reverser Ring is with either the Miranda Focabell S or Miranda Double Track Bellows Extension.    The Adapter is screwed into the front lens thread of the bellows unit, and the lens is then threaded on to the Reverser Adapter.    Exposure compensations are determined by the amount of bellows extension.    Consult Bellows instructions for the method of calculating the proper exposure factor.    Be sure to add the 3.17 Reverser Adapter factor.

A tripod is very helpful when focusing and will prevent vibrations